Indian Institute of Information Technology, Nagpur

Department of Electronics and Communication Engineering (IoT)

Semester: III (2024-2025)

Lab Record

Applied Signals and Systems (ECE 205)

Submitted by

Jayant Kumar

(BT23ECI051)

Practical No. 1

Aim: To generate and plot the following waveforms in MATLAB.

1) Sine wave

2) Cosine wave

3) Square wave

4) Triangular wave

Mathematical Equations:

1) y1(t)= sin(2πft),

f: frequency in Hz

t: time in seconds

2) y2(t)= cos(2πft),

f: frequency in Hz

t: time in seconds

Code:

% Sine Wave and Cosine Wave (f=4 Hz)

clc ; % Clear the command window

clear all; % Clear all variables

close all; % Close all figure windows

% Time vector with a step size of 0.02 seconds

t = 0:0.02:1 ;  % t represents time, defined as start : step :end

f = 4; % Set the frequency for both sine and cosine waves

y1 = sin(2*pi*f*t); % Compute the sine wave using the formula sin(2πft)

y2 = cos(2*pi*f*t); % Compute the cosine wave using the formula cos(2πft)

% Plot the Sine Wave

plot(t, y1, 'g-', 'MarkerSize', 6, 'LineWidth', 2); % Plot the sine wave with specific line properties

hold on; % Retain the sine wave plot so that the cosine wave can be plotted on the same graph

%% Plot the Cosine Wave

plot(t, y2, 'm-', 'MarkerSize', 6, 'LineWidth', 2); % Plot the cosine wave with specific line properties

% Use plot for continuous graphs; stem can be used for discrete points

% The graph shows two cycles since the frequency is set to 2

% You can change the frequency to 4 for more cycles

xlabel('Time'); % X-axis label representing time

ylabel('Amplitude', 'FontWeight', ''); % Y-axis label representing amplitude

title('Sine & Cosine Wave', 'FontSize', 14, 'FontWeight', 'Bold'); % Add title to the plot

grid on; % Enable grid lines for better visibility

legend('Sine Wave', 'Cosine Wave'); % Add a legend to differentiate between the sine and cosine waves

hold off; % Release the plot hold to allow new plots later

% Uncomment the following code to display the cosine wave on a separate graph

% figure(); % Open a new figure window for the cosine wave

% plot(t, y2, 'g*-', 'MarkerSize', 6, 'LineWidth', 2); % Plot cosine wave separately with markers

% xlabel('Time'); % Label for the x-axis in the new plot

% ylabel('Amplitude', 'FontWeight', 'Bold'); % Label for the y-axis with bold font

% title('Cosine Wave', 'FontSize', 14, 'FontWeight', 'Bold'); % Title

%% Plot the Square Wave (f=4)

z1 = 2 * ones(1, 50); % Define the high value of the square wave (2) repeated 50 times

z2 = zeros(1, 50); % Define the low value of the square wave (0) repeated 50 times

z = [z1, z2]; % Combine the high and low values to create one full cycle of the square wave

f = 4; % Number of square wave cycles to be generated

y = []; % Create an empty array to store the entire square wave signal

for i = 1:f % Loop through to create 'f' full cycles of the square wave

y = [y, z]; % Append each cycle of the square wave to the signal

end

plot(y, 'r', 'MarkerSize', 5, 'LineWidth', 3); % Plot the square wave signal

title('Square Wave', 'FontSize', 15, 'FontWeight', 'Bold'); % Add title to the square wave plot

xlabel('Time', 'FontSize', 15,'FontWeight', 'Bold'); % Add x-axis label

ylabel('Amplitude', 'FontSize', 15,'FontWeight', 'Bold'); % Add y-axis label

grid on; % Enable grid for better visualization

% Plot the Triangular Wave

figure; % Open another figure window for the triangular wave

x = 0:0.1:3; % Generate the rising part of the triangular wave

y = 2.9:-0.1:0.1; % Generate the falling part of the triangular wave

p = [x, y]; % Combine the rising and falling segments to form one complete cycle

f = 4; % Number of cycles for the triangular wave

u = []; % Initialize an empty array to store the full triangular wave signal

for i = 1:f % Loop to repeat the triangular wave for 'f' cycles

u = [u, p]; % Append each full cycle to the overall signal

end

plot(u, 'r', 'LineWidth', 3); % Plot the triangular wave signal

title('Triangular Wave', 'FontSize', 15, 'FontWeight', 'Bold'); % Add a title to the plot

xlabel('Time','FontSize', 15, 'FontWeight', 'Bold'); % Label the x-axis

ylabel('Amplitude', 'FontSize', 15,'FontWeight', 'Bold'); % Label the y-axis

grid on; % Turn on the grid for easier visualization

Output:

1.sine and cosine wave (f)

2.square wave (f=4)

3.Triangular wave (f=5)

Practical No. 2

Aim: Plot the graph with the help of equations

Mathematical Equations:

A). x(t)=3u(t)+6u(t+3)-0.5u(t-3)+6u(t+6)

B). x(t)=c.e(αt) for different values of c and α

1. C and alpha are real

. alpha>0

. alpha<0

2. C and alpha are complex

. Real part of x(t)

. Imaginary part of x(t)

C). Let x(t)=3.e(-0.5*t). Calculate y(t)= ∫ x(t).dt Plot y1(t)= ∫x(t).dt

and y2(t)= dx(t)dt for real and complex values of c and α.

% Define the unit step function

u = @(t) heaviside(t);

% Define the function y as a combination of shifted and scaled unit step functions

y = 3*u(t) + 6*u(t+3) - 0.5*u(t-3) + 6*u(t+6);

% Plot the function y over the range [-20, 20] with a red line and linewidth of 2

fplot(y, [-20, 20], 'r-', 'Linewidth', 2);

% Add a title to the plot with larger font

title('Plot for $y = 3u(t) + 6u(t+3) - 0.5u(t-3)+ 6u(t+6)$', 'interpreter', 'latex', 'FontSize', 18);

xlabel('t', 'FontSize', 18); % Label the x-axis with larger font

ylabel('Amplitude', 'FontSize', 18); % Label the y-axis with larger font

% Define symbolic variables

syms t c alpha;

% Define symbolic expression for an exponential function

y = @(t) c * exp(alpha * t);

% Substitute real values for c and alpha

y1= subs(y, {c, alpha}, {2, 5});

y2= subs(y, {c, alpha}, {2, -5});

% Substitute complex values for c and alpha

y3 = subs(y, {c, alpha}, {1 + 2j, -0.2 + 1.2j});

% Create a 2x2 subplot grid

figure;

% Plot for real exponential signal with positive alpha (in green)

subplot(2, 2, 1);

fplot(y1, 'g-', 'LineWidth', 2);

title({'Plot for $y_{1}(t) = \left(c e^{\alpha t}\right)$', 'where $\alpha > 0$ and $c=2$'}, 'interpreter', 'latex', 'FontSize', 16);

xlabel('t', 'FontSize', 16);

ylabel('Amplitude', 'FontSize', 16);

% Plot for real exponential signal with negative alpha (in green)

subplot(2, 2, 2);

fplot(y2, 'g-', 'LineWidth', 2);

title({'Plot for $y_{2}(t) = \left(c e^{\alpha t}\right)$', 'where $\alpha < 0$ and $c=2$'}, 'interpreter', 'latex', 'FontSize', 16);

xlabel('t', 'FontSize', 16);

ylabel('Amplitude', 'FontSize', 16);

% Plot for real part of the complex exponential signal (in purple)

subplot(2, 2, 3);

fplot(real(y3), [-10, 10], 'm-', 'LineWidth', 2);

xlabel('t', 'FontSize', 16);

ylabel('Real part', 'FontSize', 16);

title({'Plot for real part of $y_{3}(t) = \left(c e^{\alpha t}\right)$', 'where $c=1+2j$, $\alpha=-0.2+1.2j$'}, 'interpreter', 'latex', 'FontSize', 16);

% Plot for imaginary part of the complex exponential signal (in purple)

subplot(2, 2, 4);

fplot(imag(y3), [-10, 10], 'm-', 'LineWidth', 2);

xlabel('t', 'FontSize', 16);

ylabel('Imaginary part', 'FontSize', 16);

title({'Plot for imaginary part of $y_{3}(t) = \left(c e^{\alpha t}\right)$', 'where $c=1+2j$, $\alpha=-0.2+1.2j$'}, 'interpreter', 'latex', 'FontSize', 16);

% Super title for the whole figure with larger font

sgtitle('Exponential Signals for Real and Complex Parameters', 'FontSize', 20);

% Define symbolic variables

syms t c alpha;

% Define symbolic expressions for exponential functions

y = @(t) c * exp(alpha * t);

x = 3 * exp(-0.5 * t);

x1 = 3 * exp(0.5 * t);

x2=(1+2j)*exp((0.1+0.5j)*t);

% Calculate integrals and derivatives

y1 = int(x, t);

y2 = diff(x, t);

y3 = int(x1, t);

y4 = diff(x1, t);

y5 = int(x2,t);

y6 = diff(x2,t);

% Create a figure for all plots

figure;

% Plot integral of 3*exp(-0.5*t) (in blue)

subplot(2, 2, 1);

fplot(y1, 'b-', 'LineWidth', 2);

title({'Plot for $y{1}(t) = \int 3 e^{-0.5 t} \, dt$', 'where $c = 3$, $\alpha = -0.5$'}, 'interpreter', 'latex', 'FontSize', 16);

xlabel('t', 'FontSize', 16);

ylabel('y{1}(t)', 'FontSize', 16);

% Plot derivative of 3*exp(-0.5*t) (in blue)

subplot(2, 2, 2);

fplot(y2, 'b-', 'LineWidth', 2);

title({'Plot for $y{2}(t) = \frac{d}{dt} \left(3 e^{-0.5 t}\right)$', 'where $c = 3$, $\alpha = -0.5$'}, 'interpreter', 'latex', 'FontSize', 16);

xlabel('t', 'FontSize', 16);

ylabel('y{2}(t)', 'FontSize', 16);

% Plot integral of 3*exp(0.5*t) (in red)

subplot(2, 2, 3);

fplot(y3, 'r-', 'LineWidth', 2);

title({'Plot for $y{3}(t) = \int 3 e^{0.5 t} \, dt$', 'where $c = 3$, $\alpha = 0.5$'}, 'interpreter', 'latex', 'FontSize', 16);

xlabel('t', 'FontSize', 16);

ylabel('y{3}(t)', 'FontSize', 16);

% Plot derivative of 3*exp(0.5*t) (in red)

subplot(2, 2, 4);

fplot(y4, 'r-', 'LineWidth', 2);

title({'Plot for $y{4}(t) = \frac{d}{dt} \left(3 e^{0.5 t}\right)$', 'where $c = 3$, $\alpha = 0.5$'}, 'interpreter', 'latex', 'FontSize', 16);

xlabel('t', 'FontSize', 16);

ylabel('y{4}(t)', 'FontSize', 16);

% Super title for real exponential functions with larger font

sgtitle('Plot of the Integrals and Derivatives of Exponential Functions $3 e^{-0.5 t}$ and $3 e^{0.5 t}$', 'interpreter', 'latex', 'FontSize', 20);

% Plot integrals and derivatives for complex parameters

figure;

% Plot real part of the integral of complex exponential

subplot(2,2,1);

fplot(real(y5),[-10,10],'c-','LineWidth',2);

xlabel('t', 'FontSize', 16);

ylabel('Real Part', 'FontSize', 16);

title({'Plot for real part of $y{5}(t) = \int (1+2j) e^{(0.1+0.5j)t} \, dt$', 'where $c = 1+2j$, $\alpha = 0.1+0.5j$'}, 'interpreter', 'latex', 'FontSize', 16);

% Plot imaginary part of the integral of complex exponential (in orange)

subplot(2,2,2);

fplot(imag(y6),[-10,10],'c-','LineWidth',2);

xlabel('t', 'FontSize', 16);

ylabel('Imaginary Part', 'FontSize', 16);

title({'Plot for imaginary part of $y{5}(t) = \int (1+2j) e^{(0.1+0.5j)t} \, dt$', 'where $c = 1+2j$, $\alpha = 0.1+0.5j$'}, 'interpreter', 'latex', 'FontSize', 16);

% Plot real part of the derivative of complex exponential (in cyan)

subplot(2,2,3);

fplot(real(y6),[-10,10],'y-','LineWidth',2);

xlabel('t', 'FontSize', 16);

ylabel('Real Part', 'FontSize', 16);

title({'Plot for real part of $y{6}(t) = \frac{d}{dt} \left(1+2j) e^{(0.1+0.5j) t}\right)$', 'where $c = 1+2j$, $\alpha = 0.1+0.5j$'}, 'interpreter', 'latex', 'FontSize', 16);

% Plot imaginary part of the derivative of complex exponential (in cyan)

subplot(2,2,4);

fplot(imag(y6),[-10,10],'y-','LineWidth',2);

xlabel('t', 'FontSize', 16);

ylabel('Imaginary Part', 'FontSize', 16);

title({'Plot for imaginary part of $y{6}(t) = \frac{d}{dt} \left(1+2j) e^{(0.1+0.5j) t}\right)$', 'where $c = 1+2j$, $\alpha = 0.1+0.5j$'}, 'interpreter', 'latex', 'FontSize', 16);

% Super title for complex exponential functions with larger font

sgtitle('Plot of the derivative and integral of $y(t) = c \cdot e^{\alpha t}$ for complex values of $c$ and $\alpha$, where $c = 1 + 2j$ and $\alpha = 0.1 + 0.5j$', 'interpreter', 'latex', 'FontSize', 20);

Input Signal:

Output:

(A)

(B)

(C)

Practical No. 3

Aim: To implement a function for convolution of two signals.

Mathematical Equations:

Yn= k= -∞∞xk.hn-k

Code:

% Define the input signals and their respective index arrays

% x represents the first signal with index array nx

% h represents the second signal with index array nh

x = [5, 4, 7, 9, 11];

nx = [-3, -2, -1, 0, 1];

h = [4, 16, 7, 8];

nh = [-1, 0, 1, 2];

% Using the custom convolution function

% This function calculates the convolution of x and h manually

[y, ny] = conv_1(x, nx, h, nh);

% Display the convolution result from the custom function

disp('Custom convolved output:');

disp(y);

disp('Index range of result:');

disp(ny);

% Convolution result using built-in function

y_builtin = conv(x, h);

% Plotting the input signals and convolution results in separate subplots

figure;

% Plot the first signal x[n]

subplot(2,2,1); % Subplot at position 1 (2 rows, 2 columns)

stem(nx, x, 'LineWidth', 2, 'Color', 'r'); % Red stem plot with line width 2

xlabel('n'); % Label for horizontal axis

ylabel('x[n]'); % Label for vertical axis

title('x[n]'); % Title of this subplot

% Plot the second signal h[n]

subplot(2,2,2); % Subplot at position 2

stem(nh, h, 'LineWidth', 2, 'Color', 'g'); % Green stem plot with line width 2

xlabel('n'); % Label for horizontal axis

ylabel('h[n]'); % Label for vertical axis

title('h[n]'); % Title of this subplot

% Plot the convolution result from the custom function

subplot(2,2,3); % Subplot at position 3

stem(ny, y, 'LineWidth', 2, 'Color', 'b'); % Blue stem plot with line width 2

xlabel('n'); % Label for horizontal axis

ylabel('y[n]'); % Label for vertical axis

title('y[n] = x[n] * h[n] (Custom Function)'); % Title of this subplot

% Plot the convolution result from MATLAB's built-in function

subplot(2,2,4); % Subplot at position 4

stem(ny, y_builtin(1:length(ny)), 'LineWidth', 2, 'Color', 'm'); % Magenta stem plot

xlabel('n'); % Label for horizontal axis

ylabel('y[n]'); % Label for vertical axis

title('y[n] = x[n] * h[n] (Built-in Function)'); % Title of this subplot

% Add a main title for the entire figure

sgtitle('Convolution y[n] = x[n] * h[n]'); % Overall title for the figure

[y, ny] = conv_1(x, nx, h, nh);

% Custom function for manual convolution

function [y, ny] = conv_1(x, nx, h, nh)

% Reverse the second signal and adjust its index

h_flip = flip(h);

nh_flip = flip(-nh);

% Define the start and end of the index range for the output

ys = nx(1) + nh(1);

ye = nx(end) + nh(end);

% Initialize the output array y and its index range

y = [];

% Loop over each time shift

for j = ys:ye

% Shift the reversed nh by the current index j

nh_flip_shifted = nh_flip + j;

% Determine overlapping indices between the signals

overlap_indices = intersect(nx, nh_flip_shifted);

m = 0; % Initialize the sum for this shift

% Calculate sum of products for overlapping elements

for i = 1:length(overlap_indices)

x_value = x(find(nx == overlap_indices(i)));

h_value = h_flip(find(nh_flip_shifted == overlap_indices(i)));

m = m + x_value * h_value;

end

% Append the computed sum to the output

y = [y, m];

end

% Define the index range for the output signal

ny = ys:ye;

end

Input :

- x[n] = [5,4,7,9,11]

- h[n] = [4,16,7,8]

Output :

Practical No. 4

Aim: Continuous time convolution and computation of impulse response from

given equation.

Mathematical Equations:

yt= xt*ht=0txτht-τdτ

(B)

Yn-0.5Yn-1+1.5Yn-2=3X[n]

Code:

Part (A)

.) Continuous Time convolution of  input signal x(t) and impulse response h(t).

In this equation:

- x(t) is the input signal.

- h(t) is the impulse response.

- τ is a variable for integration which representing the time shift.

- y(t) is the result of the convolution of  x(t) and h(t).

clc;

close all;

clear all;

% Define symbolic variables for convolution

syms t z

% Create a function handle for the Heaviside step function, u(t)

u = @(t) heaviside(t);  % Returns 0 when t < 0 and 1 when t >= 0

% Define the input signal x(t) as a function handle

x = @(t) (u(t) - u(t - 5));  % x(t) is 1 within [0, 5), otherwise 0

% Define the impulse response h(t) as a function handle

h = @(t) (u(t) - u(t - 5));  % h(t) is 1 within [0, 5), otherwise 0

% Calculate the symbolic convolution of x(t) and h(t)

% z serves as a dummy variable for integration

y = int(x(z) * h(t - z), z, 0, t);

% Plot y(t), the result of the convolution, over the interval [0, 15]

fplot(y, [0, 15], 'blue', 'LineWidth', 1);  % Use fplot to plot the symbolic result

% Set the plot title

title('Symbolic Convolution y(t) = x(t) * h(t)');

% Label the x-axis as "Time t"

xlabel('Time t');

% Label the y-axis as "y(t)"

ylabel('y(t)');

Part (B)

Computation of impulse response from given differential equation

clc;

close all;

clear all;

% Number of samples to compute

N = 10;

% Coefficients for output y[n] and its past values

a = [1, -0.5, 1.5];  % Coefficients for y[n], y[n-1], y[n-2], etc.

b = [3];             % Coefficient for x[n]

% Define the impulse input signal delta[n]

delta = [1 zeros(1, N-1)];

% Define the time indices for delta[n] (input signal x[n])

nx = 0:N-1;

% Initialize the impulse response h[n] and its time indices

h = zeros(1, N);

nh = 0:N-1;

% Set initial condition for h[0] based on the input signal

h(1) = b(1) * delta(1);

% Compute the impulse response using the difference equation

for n = 2:N

% Compute the contribution from the input terms (RHS of the equation)

input_sum = 0;

for k = 1:min(n, length(b))

input_sum = input_sum + b(k) * delta(n-k+1);

end

% Compute the contribution from the output terms (LHS of the equation)

output_sum = 0;

for k = 2:min(n, length(a))  % Start from 2 to exclude a[0], which is y[n]

if (n-k+1) > 0

output_sum = output_sum + a(k) * h(n-k+1);

end

end

% Calculate h[n] based on the difference equation

h(n) = (input_sum - output_sum) / a(1);

end

% Plot the calculated impulse response h[n]

stem(nh, h, 'blue', 'LineWidth', 1);

title('Impulse Response h[n] for y[n] - 0.5y[n-1] + 1.5y[n-2] = 3x[n]');

xlabel('n');

ylabel('h[n]');

(A)

(B)

Practical No. 5

Aim: Countinuous time fourier series coefficient of given x(t)

Mathematical Equation for Continous time fourier series:

1.

ak=1T-∞∞xt.e-jkwotdt

2.

xt=-∞∞ake-jwot

where T= time period,

ak= fourier series coefficients,

k=  harmonic magnitude,

x(t)=Input signal,

w0= fundamental frequency=2π/T

Mathematical Equation for Continuous time fourier series:

1.

xn=k=0N-1ak ejkωn

2.

ak=1N k=0N-1x[n]e-jkωn

where T= time period,

N= number of samples,

ak= fourier series coefficients,

k=  harmonic magnitude,

x(t)=Input signal,

w= fundamental frequency=2π/N

Code:

- Continuous time fourier series

syms t k;

T = 8; % Period of the signal

w = (2 * pi) / T; % Angular frequency

a = (1/T) * int(exp(-1i * k * w * t), t, [-2, 2]); % Calculate the Fourier coefficient

fs = []; % Initialize frequency response array

% Calculate fs for frequency indices

for m = -200:200

if m ~= 0

b = subs(a, {k}, [m]); % Substitute k with m for non-zero indices

fs = [fs b];

else

b = (1/T) * int(1, t, [-2, 2]); % Compute fs for m = 0

fs = [fs b];

end

end

m1 = -200:200; % Frequency index range

% First figure: Stem plot of fs

figure();

stem(m1, fs, 'filled', 'LineWidth', 1, 'MarkerSize', 3, 'Color', 'b');

xlabel('w - Frequency Index', 'FontWeight', 'bold');

ylabel('Fourier coefficient-a_k', 'FontWeight', 'bold');

title('Plot of a_k', 'FontSize', 12, 'FontWeight', 'bold');

grid on;

% Second figure: Continuous plot of the signal s

figure();

s = 0; % Initialize signal s

% Summation for signal representation

for m1 = -200:200

s = s + (fs(m1 + 201) * exp(1i * m1 * w * t)); % Accumulate signal components

end

fplot(real(s), [-10, 10], 'Color', 'r', 'LineWidth', 2);

xlabel('t - Time', 'FontWeight', 'bold');

ylabel('Signal x(t)', 'FontWeight', 'bold');

title('Continuous Signal x(t)', 'FontSize', 12, 'FontWeight', 'bold');

grid on;

Input :

Output:

- Discrete time fourier series

clc;

clear all;

close all;

% Define a discrete-time periodic signal

N = 8;  % Signal period (number of samples)

n = 0:N-1;  % Time indices for one period

x = [1 2 3 4 5 6 7 8];  % Discrete-time signal values x[n]

% Define the fundamental angular frequency

w0 = 2 * pi / N;

% Initialize the Fourier Series coefficients array (Discrete Fourier Series)

a_k = zeros(1, N);

% Compute the Fourier Series coefficients a[k]

for k = 0:N-1

a_k(k+1) = (1/N) * sum(x .* exp(-1i * k * w0 * n));  % Apply DFS formula

end

% Plot the magnitude and phase of Fourier Series coefficients

figure;

subplot(2,1,1);

stem(0:N-1, abs(a_k), 'filled', 'Color', 'magenta');

title('Magnitude of Fourier Coefficients: $$|a_k| = \frac{1}{N} \sum_{n=0}^{N-1} x[n] e^{-i k w_0 n}$$', 'Interpreter', 'latex');

xlabel('k');

ylabel('Magnitude |a_k|');

grid on;

subplot(2,1,2);

stem(0:N-1, angle(a_k), 'filled', 'Color', 'cyan');

title('Phase of Fourier Coefficients: $$\angle a_k = \arg \left( \frac{1}{N} \sum_{n=0}^{N-1} x[n] e^{-i k w_0 n} \right)$$', 'Interpreter', 'latex');

xlabel('k');

ylabel('Phase \angle a_k');

grid on;

% Reconstruct the input signal from the Fourier coefficients

x_reconstructed = zeros(1, N);

for n_reconstructed = 0:N-1

for k = 0:N-1

x_reconstructed(n_reconstructed+1) = x_reconstructed(n_reconstructed+1) + a_k(k+1) * exp(1i * k * w0 * n_reconstructed);

end

end

% Plot the input signal using the reconstructed values

figure;

stem(n, real(x_reconstructed), 'filled', 'Color', 'blue');

title('Input Signal: $$x[n] = \sum_{k=0}^{N-1} a_k e^{i k w_0 n}$$', 'Interpreter', 'latex');

xlabel('n');

ylabel('x[n]');

Input:

Output:

Practical No.6

Aim: - Compute Fourier transform of x(t)

x(t) =  2t+2, -1<t<0

-2t+2, 0<t<1

Mathematical equation:

1.

ak=t=-∞∞xte-jωtdt

2.

xt=k=-∞∞akejkωt

Code:

clc;

close all;

clear all;

% Define symbolic variables for frequency and time

syms w t;

% Compute the Fourier transform of a piecewise-defined function

x = int((2*t + 2) * exp(-1i * w * t), t, -1, 0) + int((-2*t + 2) * exp(-1i * w * t), t, 0, 1);

% Initialize an array to store Fourier transform values at different frequencies

f = [];

% Define the frequency range from -10 to 10 in increments of 0.1

for b = -10:0.1:10

if b ~= 0

d = subs(x, {w}, [b]);  % Substitute the specific frequency value for w

f = [f, d];  % Append result to the Fourier transform array

else

% Calculate the Fourier transform when frequency is zero (DC component)

d = int((2*t + 2), t, -1, 0) + int((-2*t + 2), t, 0, 1);

f = [f, d];  % Append result to the Fourier transform array

end

end

% Calculate the magnitude and phase of the Fourier transform

magx = abs(f);

anx = angle(f);

% Define frequency values for plotting

b1 = -10:0.1:10;

% Create a figure window for plots

figure;

% Plot the magnitude of X(jω) versus frequency (ω)

subplot(1, 2, 1);

plot(b1, magx, 'blue', 'LineWidth', 2);  % Use blue color for magnitude plot

title('Magnitude of X(jω) vs ω', 'FontSize', 14, 'FontWeight', 'bold');

xlabel('ω', 'FontSize', 12);

ylabel('|X(jω)|', 'FontSize', 12);

grid on;

% Plot the phase of X(jω) versus frequency (ω)

subplot(1, 2, 2);

plot(b1, anx, 'green', 'LineWidth', 2);  % Use green color for phase plot

title('Phase of X(jω) vs ω', 'FontSize', 14, 'FontWeight', 'bold');

xlabel('ω', 'FontSize', 12);

ylabel('∠X(jω)', 'FontSize', 12);

grid on;

Output:

Practical No.7

Aim: De-noising of signal in Fourier domain

Mathematical Equations:

Y = awgn(X,snr) % adds white Gaussian noise to the vector signal X. This syntax assumes that the power of X is 0 dBW.

- ak=t=-∞∞x(t)e-jωtdt

- x (t)=sin(2πft)

Code:

clc;

close all;

nt = 0:0.01:1;

f = 1;

X = 1 * sin(2 * pi * f * nt);

w = (2 * pi) / length(X);

% Plot the original sine wave signal

figure();

stem(X);

title('Sine Wave);

xlabel('time');

ylabel('Amplitude');

%%%% Computing Fourier Series of Sine Wave %%%%%%%%

F = [];

for k = 0:100

s = 0;

for n = 0:100

d = X(n+1) * exp(-1i * k * w * n);

s = s + d;

end

F = [F s / length(X)];

end

% Plot Fourier Series coefficients of the sine wave

k = 0:100;

figure();

stem(k, abs(F));

title('Fourier Series of Sine Wave');

xlabel('time');

ylabel('Amplitude');

% % %%%%%%%% Adding noise to the signal %%%%%%%%%%%%%%%

y = awgn(X, 5);

% Plot the noisy signal

figure();

stem(y);

title('Noisy Signal (with AWGN)');

xlabel('Time Index(n)');

ylabel('Amplitude');

% % %%%%%%%%%%% Computing Fourier series of noisy signal %%%%%%%%%%%%%%%

F_noisy = [];

for k = 0:100

s = 0;

for n = 0:100

d = y(n+1) * exp(-1i * k * w * n);

s = s + d;

end

F_noisy = [F_noisy s / length(X)];

end

% Plot Fourier Series coefficients of the noisy signal

figure();

stem(k, abs(F_noisy));

title('Fourier Coefficients of Noisy Signal');

xlabel('Frequency Index (k)');

ylabel('Magnitude of Fourier Coefficients');

% % %%%%%%% Performing Thresholding %%%%%%%%%%%%%%%%

A = F_noisy;

A(abs(F_noisy) < 0.2) = 0;

% Plot Fourier Series coefficients after thresholding

figure();

stem(k, abs(A));

title('Thresholded Fourier Coefficients');

xlabel('Frequency Index (k)');

ylabel('Magnitude of Thresholded Coefficients');

% % %%%%%%%%%%%%%% Reconstructing Signal %%%%%%%%%%%%%%%

signal_ = [];

for n = 0:100

s = 0;

for k = 0:100

d = A(k+1) * exp(1i * k * w * n);

s = s + d;

end

signal_ = [signal_ s];

end

% Plot the reconstructed signal after thresholding

figure();

stem(signal_);

title('Reconstructed Signal after Thresholding');

xlabel('Time Index(n)');

ylabel('Amplitude');

Output: